BOM Formatting Guidelines

Rev 2_06/17/2026

BOM Formatting Guidelines

Screaming Circuits builds directly from your Bill of Materials (BOM). Your BOM serves as the link between your PCB design and the assembly process. Improper formatting can prevent a successful upload, resulting in skipped BOM lines, missing quotes, or components being incorrectly marked as DNP (Do Not Place).

For the fastest turnaround and greatest quoting accuracy, follow these BOM formatting best practices.

Avoid Special Characters

- Carriage returns and colons (:) within BOM cells can cause upload errors and may result in parts not being quoted or placed correctly.

- Use commas (,) or underscores (_) instead when needed.

- After upload, our Quoting Tool allows you to review and edit your BOM before completing your order.

Use Manufacturer Part Numbers

- Use manufacturer part numbers whenever possible rather than vendor-specific part numbers.

- Manufacturer part numbers allow us to source components from multiple suppliers, improving pricing and availability.

- Our Quoting Tool connects to 14 vendor APIs to identify the best component options.

Include Alternate Part Numbers

- Provide approved alternate or substitute manufacturer part numbers whenever possible.

- Alternate parts help reduce delays caused by shortages, limited stock, or unavailable components.

Cell Formatting

Proper cell formatting helps ensure BOM data uploads correctly and is interpreted accurately during quoting and assembly.

- Include only one part number per cell.

- Place each approved alternate part number in its own cell or designated alternate part number column.

- Information entered in the wrong cell may be skipped, lost, or interpreted incorrectly.

Reference Designator Formatting

Format reference designators using:

- A dash (-) to indicate a range

- A comma (,) to separate ranges or individual designators

Correct Examples

- C1, C2, C3, C4, C5

- C1-C5, C8-C10

- C1-C5, C8, C10

Following these guidelines minimizes upload issues, reduces manual review, and improves quoting accuracy and turnaround time